GENERAL SYSTEM

aerosonde_uav (Plant SubSystem)

air_data

function [Va, alpha, beta, rho] = calculate_env_data(V_b, V_w, pd, P) % inputs: % V_b: [3x1] body-frame ground velocity [u; v; w] (m/s) % V_w: [3x1] body-frame wind velocity [uw; vw; ww] (m/s) % pd: [1x1] down position (negative of altitude) (m) % outputs: % Va: [1x1] airspeed magnitude (m/s) % alpha: [1x1] angle of attack (rad) % beta: [1x1] sideslip angle (rad) % current height h = -pd; % temperature and pressure at altitude h T = P.T0 + P.L0 * h; p = P.p0 * (P.T0 / T)^((P.g * P.Molar) / (P.R * P.L0)); % dynamic density using ideal gas law rho = (p * P.Molar) / (P.R * T); % airspeed vector V_r = V_b - V_w; % airspeed magnitude Va = norm(V_r); % angle of attack and sideslip if Va == 0 % prevent to divide by zero alpha = 0; beta = 0; else alpha = atan2(V_r(3), V_r(1)); beta = asin(V_r(2) / Va); end end

forces_and_moments

function F_g = calculate_gravity(att, P) % inputs: % att: [3x1] euler angles [phi; theta; psi] (rad) % mass: [1x1] uav mass (kg) % g: [1x1] gravitational constant (m/s^2) % outputs: % F_g: [3x1] gravity forces [fx; fy; fz] (n) phi = att(1); theta = att(2); % project gravity to body frame F_g = P.mass * P.g * [-sin(theta); cos(theta) * sin(phi); cos(theta) * cos(phi)]; end

function [F_a, M_a] = calculate_aerodynamics(Va, alpha, beta, ... omega, delta, rho, P) % inputs: % Va: [1x1] airspeed (m/s) % alpha: [1x1] angle of attack (rad) % beta: [1x1] sideslip angle (rad) % omega: [3x1] body angular rates [p; q; r] (rad/s) % delta: [4x1] control signals [da; de; dr; dt] % rho: [1x1] air density (kg/m^3) % P: [1x1 struct] aerodynamic parameters % outputs: % F_a: [3x1] aerodynamic forces [fax; fay; faz] (n) % M_a: [3x1] aerodynamic moments [max; may; maz] (n-m) p = omega(1); q = omega(2); r = omega(3); da = delta(1); de = delta(2); dr = delta(3); % prevent division by zero at rest if Va == 0 F_a = [0; 0; 0]; M_a = [0; 0; 0]; return; end % sigmoid blending function for non-linear stall sig_num = 1 + exp(-P.M * (alpha - P.alpha0)) + ... exp(P.M * (alpha + P.alpha0)); sig_den = (1 + exp(-P.M * (alpha - P.alpha0))) * ... (1 + exp(P.M * (alpha + P.alpha0))); sigma_alpha = sig_num / sig_den; % non-linear lift coefficient CL_linear = P.C_L0 + P.C_Lalpha * alpha; CL_flat = 2 * sign(alpha) * sin(alpha)^2 * cos(alpha); CL_alpha = (1 - sigma_alpha) * CL_linear + sigma_alpha * CL_flat; % non-linear drag coefficient (quadratic) AR = (P.b^2) / P.S; CD_alpha = P.C_D0 + (CL_linear^2) / (pi * P.e * AR); % longitudinal aerodynamic coefficients cx = -CD_alpha * cos(alpha) + CL_alpha * sin(alpha); cx_q = -P.C_Dq * cos(alpha) + P.C_Lq * sin(alpha); cx_de = -P.C_Dde * cos(alpha) + P.C_Lde * sin(alpha); cz = -CD_alpha * sin(alpha) - CL_alpha * cos(alpha); cz_q = -P.C_Dq * sin(alpha) - P.C_Lq * cos(alpha); cz_de = -P.C_Dde * sin(alpha) - P.C_Lde * cos(alpha); % dynamic pressure terms q_bar = 0.5 * rho * Va^2 * P.S; c_2v = (P.c * q) / (2 * Va); b_2v_p = (P.b * p) / (2 * Va); b_2v_r = (P.b * r) / (2 * Va); % compute forces fax = q_bar * (cx + cx_q * c_2v + cx_de * de); fay = q_bar * (P.C_Y0 + P.C_Ybeta * beta + P.C_Yp * b_2v_p + ... P.C_Yr * b_2v_r + P.C_Yda * da + P.C_Ydr * dr); faz = q_bar * (cz + cz_q * c_2v + cz_de * de); % compute moments max = q_bar * P.b * (P.C_Mx0 + P.C_Mxbeta * beta + ... P.C_Mxp * b_2v_p + P.C_Mxr * b_2v_r + ... P.C_Mxda * da + P.C_Mxdr * dr); may = q_bar * P.c * (P.C_My0 + P.C_Myalpha * alpha + ... P.C_Myq * c_2v + P.C_Myde * de); maz = q_bar * P.b * (P.C_Mz0 + P.C_Mzbeta * beta + ... P.C_Mzp * b_2v_p + P.C_Mzr * b_2v_r + ... P.C_Mzda * da + P.C_Mzdr * dr); F_a = [fax; fay; faz]; M_a = [max; may; maz]; end

function [F_p, M_p] = calculate_propulsion(Va, delta, rho, P) % calculates propulsion forces and moments[cite: 1] % inputs: % Va: [1x1] airspeed (m/s) % delta:[4x1] control signals [da; de; dr; dt] % rho: [1x1] air density (kg/m^3) % P: [1x1 struct] physical parameters % outputs: % F_p: [3x1] propulsive force [fx; fy; fz] (n) % M_p: [3x1] propulsive moment [mx; my; mz] (n-m) dt = delta(4); if dt == 0 F_p = [0; 0; 0]; M_p = [0; 0; 0]; return; end v_in = P.V_max * dt; a = (rho * P.D_prop^5) / ((2 * pi)^2) * P.C_Q0; b = (rho * P.D_prop^4) / (2 * pi) * P.C_Q1 * Va + ... (P.k_Q * P.k_V) / P.R_motor; c = rho * P.D_prop^3 * P.C_Q2 * Va^2 - ... (P.k_Q / P.R_motor) * v_in + P.k_Q * P.i0; omega_p = (-b + sqrt(b^2 - 4 * a * c)) / (2 * a); t_p = (rho * P.D_prop^4) / ((2 * pi)^2) * omega_p^2 * P.C_T0 + ... (rho * P.D_prop^3) / (2 * pi) * omega_p * Va * P.C_T1 + ... rho * P.D_prop^2 * Va^2 * P.C_T2; q_p = (rho * P.D_prop^5) / ((2 * pi)^2) * omega_p^2 * P.C_Q0 + ... (rho * P.D_prop^4) / (2 * pi) * omega_p * Va * P.C_Q1 + ... rho * P.D_prop^3 * Va^2 * P.C_Q2; F_p = [t_p; 0; 0]; M_p = [-q_p; 0; 0]; end

mixer

function [F_total, M_total] = mix_forces_moments(F_g, F_p, F_a, M_p, M_a) % sums all forces and moments[cite: 1] % inputs: % F_g: [3x1] gravity force (n) % F_p: [3x1] propulsive force (n) % F_a: [3x1] aerodynamic force (n) % M_p: [3x1] propulsive moment (n-m) % M_a: [3x1] aerodynamic moment (n-m) % outputs: % F_total: [3x1] total body frame force (n) % M_total: [3x1] total body frame moment (n-m) F_total = F_g + F_p + F_a; M_total = M_p + M_a; end

dynamics

function V_b_dot = trans_dynamics(F_total, V_b, omega, P) % inputs: % F_total: [3x1] total forces [fx; fy; fz] (n) % V_b: [3x1] body velocities [u; v; w] (m/s) % omega: [3x1] angular rates [p; q; r] (rad/s) % P: [1x1 struct] physical parameters % outputs: % V_b_dot: [3x1] velocity derivatives [u_dot; v_dot; w_dot] (m/s^2) u = V_b(1); v = V_b(2); w = V_b(3); p = omega(1); q = omega(2); r = omega(3); fx = F_total(1); fy = F_total(2); fz = F_total(3); u_dot = r*v - q*w + fx / P.mass; v_dot = p*w - r*u + fy / P.mass; w_dot = q*u - p*v + fz / P.mass; V_b_dot = [u_dot; v_dot; w_dot]; end

function omega_dot = rot_dynamics(M_total, omega, P) % inputs: % M_total: [3x1] total moments [mx; my; mz] (n-m) % omega: [3x1] angular rates [p; q; r] (rad/s) % P: [1x1 struct] physical parameters % outputs: % omega_dot: [3x1] angular rate derivatives [p_dot; q_dot; r_dot] (rad/s^2) p = omega(1); q = omega(2); r = omega(3); mx = M_total(1); my = M_total(2); mz = M_total(3); p_dot = P.Gamma1*p*q - P.Gamma2*q*r + P.Gamma3*mx + P.Gamma4*mz; q_dot = P.Gamma5*p*r - P.Gamma6*(p^2 - r^2) + my / P.Jy; r_dot = P.Gamma7*p*q - P.Gamma1*q*r + P.Gamma4*mx + P.Gamma8*mz; omega_dot = [p_dot; q_dot; r_dot]; end

kinematics

function pos_dot = trans_kinematics(V_b, att) % inputs: % V_b: [3x1] body velocities [u; v; w] (m/s) % att: [3x1] euler angles [phi; theta; psi] (rad) % outputs: % pos_dot: [3x1] inertial pos derivatives [pn_dot; pe_dot; pd_dot] (m/s) u = V_b(1); v = V_b(2); w = V_b(3); phi = att(1); theta = att(2); psi = att(3); cphi = cos(phi); sphi = sin(phi); cthe = cos(theta); sthe = sin(theta); cpsi = cos(psi); spsi = sin(psi); R_b_v = [cthe*cpsi, sphi*sthe*cpsi - cphi*spsi, cphi*sthe*cpsi + sphi*spsi; cthe*spsi, sphi*sthe*spsi + cphi*cpsi, cphi*sthe*spsi - sphi*cpsi; -sthe, sphi*cthe, cphi*cthe]; pos_dot = R_b_v * [u; v; w]; end

function att_dot = rot_kinematics(omega, att) % inputs: % omega: [3x1] angular rates [p; q; r] (rad/s) % att: [3x1] euler angles [phi; theta; psi] (rad) % outputs: % att_dot: [3x1] euler angle derivatives [phi_dot; theta_dot; psi_dot] (rad/s) p = omega(1); q = omega(2); r = omega(3); phi = att(1); theta = att(2); cphi = cos(phi); sphi = sin(phi); cthe = cos(theta); tthe = tan(theta); R_rot = [1, sphi*tthe, cphi*tthe; 0, cphi, -sphi; 0, sphi/cthe, cphi/cthe]; att_dot = R_rot * [p; q; r]; end

initial condition

successive_loop_closure (Controller SubSystem)

inner_loop_pd

outer_loop_pi

function error_out = wrap_angle(error_in) % wraps any angle to strictly [-pi, pi] error_out = mod(error_in + pi, 2*pi) - pi; end

function psi_wrapped = wrap_plot(psi) psi_wrapped = mod(psi + pi, 2*pi) - pi; end

guidance (Guidance SubSystem)

function [chi_cmd, alt_cmd, Va_cmd, path_cmd, theta_dot] = ... generate_trajectory(pn_pe_pd, theta, traj_type) %% extract current UAV position pn = pn_pe_pd(1); pe = pn_pe_pd(2); A = 300; V_uav = 15.0; N_center = 500; E_center = 500; if traj_type == 1 % figure-8 % theta represents the carrot's current position n_carrot = N_center + A * sin(theta) * cos(theta); e_carrot = E_center + A * sin(theta); alt_carrot = 100; % calculate path gradients at the carrot dN_dtheta = A * cos(2 * theta); dE_dtheta = A * cos(theta); else % helix n_carrot = N_center + A * cos(theta); e_carrot = E_center + A * sin(theta); alt_carrot = 100 + (13.0 * theta); dN_dtheta = -A * sin(theta); dE_dtheta = A * cos(theta); end %% virtual target feedback loop % calculate actual physical distance between UAV and carrot d_actual = sqrt((n_carrot - pn)^2 + (e_carrot - pe)^2); % enforce a strict lookahead tether (e.g., exactly 40 meters) L_desired = 40.0; % carrot speed controller: % if d_actual < 40, carrot speeds up % if d_actual > 40, it slows down k_carrot = 1.5; % tuning gain for target speed V_carrot = V_uav + k_carrot * (L_desired - d_actual); % saturation if V_carrot > 25.0 V_carrot = 25.0; % max sprint speed elseif V_carrot < 0.0 V_carrot = 0.0; % stop and wait for UAV end % convert linear carrot speed to angular geometric speed gradient = sqrt(dN_dtheta^2 + dE_dtheta^2); theta_dot = V_carrot / gradient; %% guidance law chi_los = atan2(e_carrot - pe, n_carrot - pn); chi_q = atan2(dE_dtheta, dN_dtheta); error_los = mod((chi_los - chi_q) + pi, 2*pi) - pi; % heading command = path tangent + proportional correction k_path = 1.2; chi_cmd = chi_q + (k_path * error_los); alt_cmd = alt_carrot; Va_cmd = V_uav; % output for 3D visualization path_cmd = [n_carrot; e_carrot; alt_carrot; traj_type]; end

wind_model (Disturbance SubSystem)

function Vw_body = rotate_wind(phi_theta_psi, Vw_inertial) phi = phi_theta_psi(1); theta = phi_theta_psi(2); psi = phi_theta_psi(3); % Rotation Matrix from Inertial to Body Frame (R_v^b) R_yaw = [cos(psi), sin(psi), 0; -sin(psi), cos(psi), 0; 0, 0, 1]; R_pitch = [cos(theta), 0, -sin(theta); 0, 1, 0; sin(theta), 0, cos(theta)]; R_roll = [1, 0, 0; 0, cos(phi), sin(phi); 0, -sin(phi), cos(phi)]; R_v_to_b = R_roll * R_pitch * R_yaw; % Total body frame wind (Ready for gusts to be added later) Vw_body = R_v_to_b * Vw_inertial; end

visualization (Visualization SubSystem)

function draw_uav_3d(pn_pe_pd, phi_theta_psi, path_cmd) coder.extrinsic('update_uav_plot'); update_uav_plot(pn_pe_pd, phi_theta_psi, path_cmd); end

MATLAB CODE

load_uav_params.m

% ========================================================================= % AEROSONDE UAV PARAMETERS % ========================================================================= %% --- GRAVITY AND MASS --- P.g = 9.81; % Gravitational constant (m/s^2) P.mass = 11.0; % Aircraft mass (kg) %% --- MOMENTS AND PRODUCTS OF INERTIA --- % These determine the aircraft's resistance to angular acceleration. P.Jx = 0.824; % Roll moment of inertia (kg-m^2) P.Jy = 1.135; % Pitch moment of inertia (kg-m^2) P.Jz = 1.759; % Yaw moment of inertia (kg-m^2) P.Jxz = 0.120; % Product of inertia (kg-m^2) %% --- INERTIA KINEMATIC CONSTANTS --- % Pre-computed constants used in the rigid-body rotational equations. Gamma = P.Jx*P.Jz - P.Jxz^2; P.Gamma1 = P.Jxz*(P.Jx - P.Jy + P.Jz) / Gamma; P.Gamma2 = (P.Jz*(P.Jz - P.Jy) + P.Jxz^2) / Gamma; P.Gamma3 = P.Jz / Gamma; P.Gamma4 = P.Jxz / Gamma; P.Gamma5 = (P.Jz - P.Jx) / P.Jy; P.Gamma6 = P.Jxz / P.Jy; P.Gamma7 = ((P.Jx - P.Jy)*P.Jx + P.Jxz^2) / Gamma; P.Gamma8 = P.Jx / Gamma; %% --- AIRCRAFT GEOMETRY --- P.S = 0.55; % Wing planform area (m^2) P.b = 2.9; % Wingspan (m) P.c = 0.19; % Mean aerodynamic chord (m) %% --- NONLINEAR STALL AND DRAG PARAMETERS --- P.M = 50; % Stall blending function slope P.alpha0 = 0.47; % Stall angle of attack (rad) P.e = 0.9; % Oswald efficiency factor %% --- LONGITUDINAL AERODYNAMIC COEFFICIENTS --- % Subscripts: % 0 = zero angle of attack % alpha = angle of attack derivative % q = pitch-rate derivative % de = elevator deflection derivative % Lift P.C_L0 = 0.23; P.C_Lalpha = 5.61; P.C_Lq = 7.95; P.C_Lde = 0.13; % Drag P.C_D0 = 0.0424; P.C_Dalpha = 0.132; P.C_Dq = 0; P.C_Dde = 0.0135; P.C_Dp = 0.043; % Pitch moment P.C_My0 = 0.0135; P.C_Myalpha = -2.74; P.C_Myq = -38.21; P.C_Myde = -0.99; %% --- LATERAL AERODYNAMIC COEFFICIENTS --- % Subscripts: % beta = sideslip angle % p = roll rate % r = yaw rate % da = aileron deflection % dr = rudder deflection % Side force P.C_Y0 = 0; P.C_Ybeta = -0.83; P.C_Yp = 0; P.C_Yr = 0; P.C_Yda = 0.075; P.C_Ydr = 0.19; % Roll moment P.C_Mx0 = 0; P.C_Mxbeta = -0.13; P.C_Mxp = -0.51; P.C_Mxr = 0.25; P.C_Mxda = 0.17; P.C_Mxdr = 0.0024; % Yaw moment P.C_Mz0 = 0; P.C_Mzbeta = 0.073; P.C_Mzp = -0.069; P.C_Mzr = -0.095; P.C_Mzda = -0.011; P.C_Mzdr = -0.069; %% --- PROPULSION PARAMETERS (MOTOR & BATTERY) --- P.V_max = 44.4; % Maximum battery voltage (V) P.k_V = 0.0659; % Motor constant (V·s/rad) P.k_Q = 0.0659; % Motor torque constant (N·m/A) P.R_motor = 0.042; % Motor winding resistance (Ohms) P.i0 = 1.5; % No-load current (A) %% --- PROPELLER AERODYNAMIC COEFFICIENTS --- % Torque coefficient polynomial P.C_Q0 = 0.005230; P.C_Q1 = 0.004970; P.C_Q2 = -0.01664; % Thrust coefficient polynomial P.C_T0 = 0.09357; P.C_T1 = -0.06044; P.C_T2 = -0.1079; P.D_prop = 0.508; % Propeller diameter (m) %% --- ATMOSPHERIC PARAMETERS --- P.T0 = 288.15; % Standard temperature at sea level (K) P.p0 = 101325; % Standard pressure at sea level (Pa) P.L0 = -0.0065; % Temperature lapse rate (K/m) P.R = 8.31432; % Universal gas constant (J/(mol·K)) P.Molar = 0.0289644; % Molar mass of air (kg/mol) P.rho = 1.268; % Air density (kg/m^3)

compute_tf_coeffs.m

function TF = compute_tf_coeffs(Va, alpha, theta, de, dt, P) % computes longitudinal and lateral transfer function coefficients % inputs: % Va: [1x1] trim airspeed (m/s) % alpha: [1x1] trim angle of attack (rad) % theta: [1x1] trim pitch angle (rad) % de: [1x1] trim elevator (rad) % dt: [1x1] trim throttle (unitless) % P: [1x1 struct] physical parameters % outputs: % TF: [1x1 struct] transfer function coefficients % 1. dynamic pressure and air density (assuming sea level for trim) rho = P.rho; % 2. lateral coefficients (roll subsystem) cp_p = P.Gamma3 * P.C_Mxp + P.Gamma4 * P.C_Mzp; cp_da = P.Gamma3 * P.C_Mxda + P.Gamma4 * P.C_Mzda; TF.a_phi1 = -0.5 * rho * Va^2 * P.S * P.b * cp_p * (P.b / (2 * Va)); TF.a_phi2 = 0.5 * rho * Va^2 * P.S * P.b * cp_da; % 3. longitudinal coefficients (pitch subsystem) TF.a_theta1 = -(rho * Va^2 * P.c * P.S / (2 * P.Jy)) * P.C_Myq * (P.c / (2 * Va)); TF.a_theta2 = -(rho * Va^2 * P.c * P.S / (2 * P.Jy)) * P.C_Myalpha; TF.a_theta3 = (rho * Va^2 * P.c * P.S / (2 * P.Jy)) * P.C_Myde; % 4. longitudinal coefficients (airspeed subsystem) eps_num = 1e-4; % numerical derivative of thrust w.r.t airspeed (dTp_dVa) [f_p_plus, ~] = get_propulsion(Va + eps_num, dt, rho, P); [f_p_minus, ~] = get_propulsion(Va - eps_num, dt, rho, P); dtp_dva = (f_p_plus(1) - f_p_minus(1)) / (2 * eps_num); % numerical derivative of thrust w.r.t throttle (dTp_ddt) [f_p_plus, ~] = get_propulsion(Va, dt + eps_num, rho, P); [f_p_minus, ~] = get_propulsion(Va, dt - eps_num, rho, P); dtp_ddt = (f_p_plus(1) - f_p_minus(1)) / (2 * eps_num); % linear drag coefficient derivative ar = (P.b^2) / P.S; cd_alpha = 2 * (P.C_L0 + P.C_Lalpha * alpha) * P.C_Lalpha / (pi * P.e * ar); TF.a_V1 = (rho * Va * P.S / P.mass) * (P.C_D0 + cd_alpha * alpha + P.C_Dde * de) - (1 / P.mass) * dtp_dva; TF.a_V2 = (1 / P.mass) * dtp_ddt; TF.a_V3 = P.g * cos(theta - alpha); end function [F_p, M_p] = get_propulsion(Va, dt, rho, P) % localized propulsion logic for numerical derivative calculations if dt == 0 F_p = [0;0;0]; M_p = [0;0;0]; return; end v_in = P.V_max * dt; a = (rho * P.D_prop^5) / ((2 * pi)^2) * P.C_Q0; b = (rho * P.D_prop^4) / (2 * pi) * P.C_Q1 * Va + (P.k_Q * P.k_V) / P.R_motor; c = rho * P.D_prop^3 * P.C_Q2 * Va^2 - (P.k_Q / P.R_motor) * v_in + P.k_Q * P.i0; omega_p = (-b + sqrt(b^2 - 4 * a * c)) / (2 * a); t_p = (rho*P.D_prop^4)/((2*pi)^2)*omega_p^2*P.C_T0 + (rho*P.D_prop^3)/(2*pi)*omega_p*Va*P.C_T1 + rho*P.D_prop^2*Va^2*P.C_T2; q_p = (rho*P.D_prop^5)/((2*pi)^2)*omega_p^2*P.C_Q0 + (rho*P.D_prop^4)/(2*pi)*omega_p*Va*P.C_Q1 + rho*P.D_prop^3*Va^2*P.C_Q2; F_p = [t_p; 0; 0]; M_p = [-q_p; 0; 0]; end

update_uav_plot.m

function update_uav_plot(pn_pe_pd, phi_theta_psi, path_cmd) % persistent variables keep the figure open and update it every step persistent fig_handle patches traj_actual ... traj_desired p_history last_update_time % fps limiter (caps graphics to ~25 FPS to prevent lag) if isempty(last_update_time) last_update_time = tic; elseif toc(last_update_time) < 0.04 return; else last_update_time = tic; end % extract states pn = pn_pe_pd(1); pe = pn_pe_pd(2); alt = -pn_pe_pd(3); phi = phi_theta_psi(1); theta = phi_theta_psi(2); psi = phi_theta_psi(3); % initialize figure if isempty(fig_handle) ¦¦ ~isvalid(fig_handle) fig_handle = figure('Name', '3D UAV Visualization', 'Color', ... 'w', 'NumberTitle', 'off'); ax = axes('Parent', fig_handle, 'Color', [0.98 0.98 0.98]); hold(ax, 'on'); grid(ax, 'on'); view(ax, 3); % scale to fit the figure-8 and helix axis(ax, [100 900 100 900 -100 500]); xlabel(ax, 'East (m)', 'FontWeight', 'bold'); ylabel(ax, 'North (m)', 'FontWeight', 'bold'); zlabel(ax, 'Altitude (m)', 'FontWeight', 'bold'); % generate and plot the full desired path if length(path_cmd) >= 4 traj_type = path_cmd(4); A = 300; N_center = 500; E_center = 500; if traj_type == 1 % figure-8 theta_full = linspace(0, 2*pi, 200); % full geometric loop N_full = N_center + A * sin(theta_full) .* cos(theta_full); E_full = E_center + A * sin(theta_full); Alt_full = 100 * ones(size(theta_full)); else % helix theta_full = linspace(0, 8*pi, 400); % 4 spiral loops N_full = N_center + A * cos(theta_full); E_full = E_center + A * sin(theta_full); Alt_full = 100 + (13.0 * theta_full); end % draw the full static path (red dashed line) plot3(ax, E_full, N_full, Alt_full, 'r--', 'LineWidth', ... 1.5, 'DisplayName', 'Desired Path'); % initialize the moving target "carrot" (red dot) traj_desired = plot3(ax, path_cmd(2), path_cmd(1), path_cmd(3), ... 'ro', 'MarkerSize', 6, ... 'MarkerFaceColor', 'r', ... 'DisplayName', 'Target'); end % plot actual path (blue) traj_actual = plot3(ax, pe, pn, alt, 'b-', 'LineWidth', 1.5, ... 'DisplayName', 'Actual Path'); p_history = [pe; pn; alt]; % enforce only the trajectory lines in the legend legend(ax, 'Location', 'northeast'); % visualize the uav S = 3.0; % size scale uav_color = [0.6 0.6 0.6]; % grey color % vertices (body frame: x-forward, y-right, z-down) V_fuse = [1 0 0; 0 0.1 0.1; 0 -0.1 0.1; 0 0.1 -0.1; 0 -0.1 -0.1; -3 0 0] * S; F_fuse = [1 2 4; 1 4 3; 1 3 5; 1 5 2; 6 2 4; 6 4 3; 6 3 5; 6 5 2]; V_wing = [ 0 2 0; -0.5 2 0; -0.5 -2 0; 0 -2 0] * S; F_wing = [1 2 3 4]; V_htail = [-2.5 0.7 0; -3 0.7 0; -3 -0.7 0; -2.5 -0.7 0] * S; F_htail = [1 2 3 4]; V_vtail = [-2.5 0 0; -3 0 0; -3 0 -1; -2.5 0 -1] * S; F_vtail = [1 2 3 4]; % save geometry patches.v_fuse = V_fuse; patches.v_wing = V_wing; patches.v_htail = V_htail; patches.v_vtail = V_vtail; % render (HandleVisibility = 'off' keeps them out of the legend) patches.h_fuse = patch(ax, 'Vertices', V_fuse, 'Faces', F_fuse, ... 'FaceColor', uav_color, 'EdgeColor', 'k', ... 'HandleVisibility', 'off'); patches.h_wing = patch(ax, 'Vertices', V_wing, 'Faces', F_wing, ... 'FaceColor', uav_color, 'EdgeColor', 'k', ... 'HandleVisibility', 'off'); patches.h_htail = patch(ax, 'Vertices', V_htail, 'Faces', F_htail, ... 'FaceColor', uav_color, 'EdgeColor', 'k', ... 'HandleVisibility', 'off'); patches.h_vtail = patch(ax, 'Vertices', V_vtail, 'Faces', F_vtail, ... 'FaceColor', uav_color, 'EdgeColor', 'k', ... 'HandleVisibility', 'off'); else % update actual trajectory path line p_history(:, end+1) = [pe; pn; alt]; set(traj_actual, 'XData', p_history(1,:), ... 'YData', p_history(2,:), ... 'ZData', p_history(3,:)); % update target carrot position if length(path_cmd) >= 3 set(traj_desired, 'XData', path_cmd(2), ... 'YData', path_cmd(1), ... 'ZData', path_cmd(3)); end end % update uav 3d orientation R_roll = [1 0 0; 0 cos(phi) -sin(phi); 0 sin(phi) cos(phi)]; R_pitch = [cos(theta) 0 sin(theta); 0 1 0; -sin(theta) 0 cos(theta)]; R_yaw = [cos(psi) -sin(psi) 0; sin(psi) cos(psi) 0; 0 0 1]; R_body2ned = R_yaw * R_pitch * R_roll; % map NED body coordinates to plot axes (x=east, y=north, z=up) C_ned2plot = [0 1 0; 1 0 0; 0 0 -1]; R_total = C_ned2plot * R_body2ned; tf = @(v) (R_total * v')' + [pe, pn, alt]; % apply math to graphics objects set(patches.h_fuse, 'Vertices', tf(patches.v_fuse)); set(patches.h_wing, 'Vertices', tf(patches.v_wing)); set(patches.h_htail, 'Vertices', tf(patches.v_htail)); set(patches.h_vtail, 'Vertices', tf(patches.v_vtail)); drawnow limitrate; end

STM32

- Configuration

  - Project Manager

  - Clock Configuration

  - Pinout & Configuration

- Programming